Slovenia  (15-06-2005)

A study has been made and it is proposed to plan 6 multiplexes in the 470 – 862 MHz band and one multiplex in VHF Band III. After the analog networks will cease the following usage of the multiplexes is planned:

- One national SFN for coverage of 95% of the population,

- Three national networks consisted of a mixture of MFNs and regional SFNs for coverage of at least 80% of the population,

- Two MFNs or SFNs are planned in every regional area for regional programs

- Six small networks (diameter approximately 25km) consisted of a mixture of MFNs (reuse of the same channel as much as possible) and SFNs in VHF Band III for mobile reception in “ hot spot ” areas in the beginning

- Some local transmitters.

For both MFNs and SFNs 64 QAM 8k system is favored. For mobile reception QPSK system probably will be used. The long-term aim is to transfer as many MFNs as possible to SFNs and to ensure portable indoor and possibly mobile reception for the whole population. No decisions have been made on the timing to shutdown the analog transmitters. The introduction of DVB-T will probably be possible in 2006. Public broadcaster started in year 2000 with DVB-T test transmissions (medium power transmitter on channel 37) to cover the capital city of Slovenia – Ljubljana with its surroundings. The service areas of the SFN networks are planned to cover “ hot spot ” areas.

Regarding DVB-H transmission Slovenia had a trial in Radenci (north-eastern part of Slovenia). DVB-T and DVB-H were transmitted simultaneously on the 32. channel in hierarchically mode.

Receivers were handhelds and they worked properly when they were close to the transmitter of 10 W of ERP.

The legislation concerning DVB-T is expected to be prepared at the end of this year.

Second session of the RRC-06 conference will be here soon so process of coordination on bilateral and multilateral basis still goes on.