T-DAB:

Turkish Radio-Television Corporation (TRT) has began T-DAB test transmissions on 01 March 2002 in Ankara (the Capital). One transmitter covers the Ankara city center and reaches approximately 2 million people.

The transmitter power is 250 Watt with 10 dB antenna gain. The regulatory procedures for frequency allocation are completed. The frequency used in the project is 225.648 MHz in band 12 B. Radio – 3 that has classical music content and Radio – 2 (TRT – FM) that has popular Turkish music content are broadcasted via this T-DAB transmitter.

TRT started SI (Service Information) PAD (Programme associated data) transmissions including the broadcaster's name, channel's name and programme type and non-PAD transmissions such as the information about the weather and road/traffic. It is planned to update the information of the flash news, weather, road, and foreign currency/exchange from the TRT's website automatically.

T-DAB test transmissions will be enlarged to a SFN (Single Frequency Network) in early 2004, to reach 3,5 million listeners with the addition of a 1kWatt new transmitter.

Please kindly find below the System Parameters of DAB Transmitter and T-DAB Pilot Project Outline.

System Parameters of DAB Transmitter:

| Source Coding Modulation Frequency Transmitter Power | Radio 2 (TRT FM)..……… 256 Kbit/s (Stereo) Radio 3 ……………………256 Kbit/s (Stereo) COFDM Mode I VHF III (12B) = 225.648 Mhz 250 W |
| --- | --- |
| Emission Point Antenna Output Power Antenna Height Antenna Polarization Antenna Type Antenna Gain | TRT Oran; 32E50.00, 39N50.03 ERP  34 dBW + 45 m (a.g.l) Vertical Dipole antenna  10 dBi |

Radiation  Patterns

-3 dB

16 deg.

N

0 dB

- 6 dB

Vertical                                                     Horizontal